# Binary to Excess-3 Converter

# Theory

Binary to Excess-3 converter is a combinational logic circuit that converts a 4-bit Binary Coded Decimal (BCD) input into its corresponding Excess-3 code. The Excess-3 code is obtained by adding 3 (0011) to the BCD input. It is a non-weighted code and is self-complementing, which makes it useful in digital systems.

# Task

Design and implement a Binary to Excess-3 converter using logic gates and verify the results with a truth table.

# Mathematical Rule

Excess-3 Output = Binary Input + 3 (0011)

# Truth Table:

| Binary Input |  |  |  | Access 3 Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | W | X | Y | Z |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |

# Working

The converter works by adding binary 0011 to the BCD input. For example, if the input is 0010 (decimal 2), adding 0011 results in 0101 (decimal 5 in Excess-3). The logic is implemented using combinational circuits derived from Karnaugh maps.

# Diagram:

# Procedure

- Write the truth table for BCD to Excess-3 conversion.

- Simplify the Boolean expressions using K-maps.

- Implement the circuit using logic gates (AND, OR, NOT).

- Connect the circuit on a breadboard using ICs.

- Apply inputs and verify outputs.

# Hardware Components

IC 7408 (AND Gates) IC 7432 (OR Gates) IC 7404 (NOT Gates)

Breadboard, wires, power supply

# Conclusion

The Binary to Excess-3 converter successfully converts BCD inputs into Excess-3 outputs. It is widely used due to its simplicity and self-complementing property.

# Learning Outcomes

- Understand code conversion in digital systems

- Learn implementation using logic gates

- Gain knowledge of K-map simplification

- Understand practical circuit building

# Task

Design and implement a Binary to Excess-3 converter using logic gates and verify the results with a truth table.